Apollo 11 Command and Service Module (CSM)

NSSDC ID: 69-059A

Other Name(s)

- Apollo 11 CSM

- CSM-107

- Columbia

- 04039

Launch Date/Time: 1969-07-16 at 13:32:00 UTC
On-orbit dry mass: 28801.00 kg

Description

Apollo 11 was the first mission in which humans walked on the lunar surface and returned to Earth. On 20 July 1969 two astronauts (Apollo 11 Commander Neil A. Armstrong and LM pilot Edwin E. "Buzz" Aldrin Jr.) landed in Mare Tranquilitatis (the Sea of Tranquility) on the Moon in the Lunar Module (LM) while the Command and Service Module (CSM) (with CM pilot Michael Collins) continued in lunar orbit. During their stay on the Moon, the astronauts set up scientific experiments, took photographs, and collected lunar samples. The LM took off from the Moon on 21 July and the astronauts returned to Earth on 24 July.

Mission Profile

After launch on Saturn V SA-504 on 16 July 1969 at 13:32 UT (9:32 a.m. EDT) from pad 39A of Kennedy Space Center, Apollo 11 entered Earth orbit. After 1 1/2 Earth orbits, the S-IVB stage was re-ignited at 16:16:16 UT for a translunar injection burn of 5 minutes, 48 seconds putting the spacecraft on course for the Moon. The CSM separated from the S-IVB stage containing the LM 33 minutes later, turned around and docked with the LM at 16:56:03 UT. About an hour and 15 minutes later the S-IVB stage was injected into heliocentric orbit. During translunar coast a color TV transmission was made from Apollo 11 and on 17 July a 3-second mid-course correction burn of the main engine was performed. Lunar orbit insertion was achieved on 19 July at 17:21:50 UT by a retrograde firing of the main engine for 357.5 seconds while the spacecraft was behind the Moon and out of contact with Earth. A later 17 second burn circularized the orbit. On 20 July Armstrong and Aldrin entered the LM for final checkout. At 18:11:53 the LM and CSM separated. After a visual inspection by Collins, the LM descent engine fired for 30 seconds at 19:08 UT, putting the craft into a descent orbit with a closest approach 14.5 km above the Moon's surface. At 20:05 the LM descent engine fired for 756.3 seconds and descent to the lunar surface began.

The LM landed at 20:17:40 UT (4:17:40 p.m. EDT) in Mare Tranquilitatis (the Sea of Tranquility), Armstrong reporting, "Houston, Tranquility Base here - the Eagle has landed." Armstrong stepped onto the lunar surface at 02:56:15 UT on 21 July (10:56:15 p.m. July 20 EDT) stating, "That's one small step for man, one giant leap for mankind", and Aldrin followed 19 minutes later. The astronauts deployed the EASEP and other instruments, took photographs, and collected 21.7 kg of lunar rock and soil. The astronauts traversed a total distance of about 250 meters. The EVA ended at 5:11:13 UT when the astronauts returned to the LM and closed the hatch.

The LM lifted off from the Moon at 17:54:01 UT on 21 July after 21 hours, 36 minutes on the lunar surface. After docking with the CSM at 21:34:00 UT, the LM was jettisoned into lunar orbit at 00:01:01 UT on 22 July. Transearth injection began at 04:54:42 UT on 22 July with a 2 1/2 minute firing of the CSM main engine. A mid-course correction was made later on 22 July. The CM separated from the SM at 16:21:13 UT on 24 July. Apollo 11 splashed down in the Pacific Ocean on 24 July 1969 at 16:50:35 UT (12:50:35 p.m. EDT) after a mission elapsed time of 195 hrs, 18 mins, 35 secs. The splashdown point was 13 deg 19 min N, 169 deg 9 min W, 400 miles SSW of Wake Island and 24 km (15 mi) from the recovery ship USS Hornet.

The performance of the spacecraft was excellent throughout the mission. The primary mission goal of landing astronauts on the Moon and returning them to Earth was achieved. Armstrong was a civilian on his second spaceflight (he'd previously flown on Gemini 8), Aldrin was a USAF Colonel on his second spaceflight (Gemini 12), Collins was a USAF Lt. Colonel also on his second flight (Gemini 10). The backup crew for this mission was Jim Lovell, Fred Haise, and William Anders. The Apollo 11 Command Module is on display at the National Air and Space Museum in Washington, D.C.

Spacecraft and Subsystems

As the name implies, the Command and Service Module (CSM) was comprised of two distinct units: the Command Module (CM), which housed the crew, spacecraft operations systems, and re-entry equipment, and the Service Module (SM) which carried most of the consumables (oxygen, water, helium, fuel cells, and fuel) and the main propulsion system. The total length of the two modules attached was 11.0 meters with a maximum diameter of 3.9 meters. Block II CSM's were used for all the crewed Apollo missions. The Apollo 11 CSM mass of 28,801 kg was the launch mass including propellants and expendables, of this the Command Module (CM 107) had a mass of 5557 kg and the Service Module (SM 107) 23,244 kg.

Telecommunications included voice, television, data, and tracking and ranging subsystems for communications between astronauts, CM, LM, and Earth. Voice contact was provided by an S-band uplink and downlink system. Tracking was done through a unified S-band transponder. A high gain steerable S-band antenna consisting of four 79-cm diameter parabolic dishes was mounted on a folding boom at the aft end of the SM. Two VHF scimitar antennas were also mounted on the SM. There was also a VHF recovery beacon mounted in the CM. The CSM environmental control system regulated cabin atmosphere, pressure, temperature, carbon dioxide, odors, particles, and ventilation and controlled the temperature range of the electronic equipment.

Command Module

The CM was a conical pressure vessel with a maximum diameter of 3.9 m at its base and a height of 3.65 m. It was made of an aluminum honeycomb sandwhich bonded between sheet aluminum alloy. The base of the CM consisted of a heat shield made of brazed stainless steel honeycomb filled with a phenolic epoxy resin as an ablative material and varied in thickness from 1.8 to 6.9 cm. At the tip of the cone was a hatch and docking assembly designed to mate with the lunar module. The CM was divided into three compartments. The forward compartment in the nose of the cone held the three 25.4 m diameter main parachutes, two 5 m drogue parachutes, and pilot mortar chutes for Earth landing. The aft compartment was situated around the base of the CM and contained propellant tanks, reaction control engines, wiring, and plumbing. The crew compartment comprised most of the volume of the CM, approximately 6.17 cubic meters of space. Three astronaut couches were lined up facing forward in the center of the compartment. A large access hatch was situated above the center couch. A short access tunnel led to the docking hatch in the CM nose. The crew compartment held the controls, displays, navigation equipment and other systems used by the astronauts. The CM had five windows: one in the access hatch, one next to each astronaut in the two outer seats, and two forward-facing rendezvous windows. Five silver/zinc-oxide batteries provided power after the CM and SM detached, three for re-entry and after landing and two for vehicle separation and parachute deployment. The CM had twelve 420 N nitrogen tetroxide/hydrazine reaction control thrusters. The CM provided the re-entry capability at the end of the mission after separation from the Service Module.

Service Module

The SM was a cylinder 3.9 meters in diameter and 7.6 m long which was attached to the back of the CM. The outer skin of the SM was formed of 2.5 cm thick aluminum honeycomb panels. The interior was divided by milled aluminum radial beams into six sections around a central cylinder. At the back of the SM mounted in the central cylinder was a gimbal mounted re-startable hypergolic liquid propellant 91,000 N engine and cone shaped engine nozzle. Attitude control was provided by four identical banks of four 450 N reaction control thrusters each spaced 90 degrees apart around the forward part of the SM. The six sections of the SM held three 31-cell hydrogen oxygen fuel cells which provided 28 volts, two cryogenic oxygen and two cryogenic hydrogen tanks, four tanks for the main propulsion engine, two for fuel and two for oxidizer, and the subsystems the main propulsion unit. Two helium tanks were mounted in the central cylinder. Environmental control radiator panels were spaced around the top of the cylinder and electrical power system radiators near the bottom.

Apollo Program

The Apollo program included a large number of uncrewed test missions and 12 crewed missions: three Earth orbiting missions (Apollo 7, 9 and Apollo-Soyuz), two lunar orbiting missions (Apollo 8 and 10), a lunar swingby (Apollo 13), and six Moon landing missions (Apollo 11, 12, 14, 15, 16, and 17). Two astronauts from each of these six missions walked on the Moon (Neil Armstrong, Edwin Aldrin, Charles Conrad, Alan Bean, Alan Shepard, Edgar Mitchell, David Scott, James Irwin, John Young, Charles Duke, Gene Cernan, and Harrison Schmitt), the only humans to have set foot on another solar system body. Total funding for the Apollo program was approximately $20,443,600,000

http://nssdc.gsfc.nasa.gov/planetary/lunar/apollo11info.html